Front and Rear Diff Plug Sizes

Front Diff: 
Up to differential serial number 254845: Tighten the hexagonal drive, (8 mm hex), drain plug to 54 Nm (40 lb-ft).

From differential serial number 254836: Tighten the 3/8'' square drive drain plug to 28 Nm (21 lb-ft). 

The Filler plug should be tightened to 34Nm (25 lb-ft).

Rear Diff: 
Up to differential serial number 254325: Tighten the hexagonal drive drain plug to 54 Nm (40 lb-ft).

From differential serial number 254326: Tighten the 3/8'' square drive drain plug to 28 Nm (21 lb-ft). 

The Filler plug should be tightened to 34Nm (25 lb-ft). 


Transfer Case oil drain: (uses Special Transfer Box Oil, Shell TF 0753, (LRN0753) instead of XO Diff Oil). The Transfer box used 8mm hex head for both Drain and Fill. 

Transfer Box Drain and Filler Plug should both be tightened to 22Nm (16 lb-ft).